# Factory Bring-Up Procedure – Car Radio

Generated: 2025-09-27 13:23

# Factory Bring-Up Procedure for Car Radio Board

## 4. Voltage Rail Checks

Table – Voltage Rails to Check

| Item | Pin | Net | Expected | Probe Points | Measured / P/F |
| --- | --- | --- | --- | --- | --- |
| a. | C47-1 | 327+5V | 5.00 V (±0.10 V) | C47-1, C48-1, C49-1, C72-1, C74-1, R17-2, R23-2, U12-4, U12-6, U6-14, U6-15, U6-16, U7-10, U7-2, U7-9, U8-5 |  |
| b. | C1-1 | 327+3V3 | 3.30 V (±0.10 V) | C1-1, C2-1, C3-1, C4-1, C5-1, C50-2, C51-2, C52-2, C53-1, C54-1, C58-2, C59-2, C6-2, C68-1, C69-1, C7-2, C70-2, C71-1, C8-2, L10-2, R1-2, R19-2, R3-2, R33-1, R5-2, R6-2, R7-2, R9-2, U1-1, U1-13, U1-19, U1-32, U1-48, U1-64, U10-5, U10-8, U11-7, U11-8, U13-15, U13-2, U5-3, U5-8 |  |
| c. | P1-1 | 317+3V3 | 3.30 V (±0.10 V) | P1-1, P2-2, P2-4 |  |
| d. | C12-1 | 327+3V3_RF | 3.30 V (±0.10 V) | C12-1, C25-1, C26-1, C27-1, C28-1, C34-1, C73-2, U12-1, U12-2, U2-14, U2-24, U2-3, U3-6, U4-6 |  |

## 1. **Prepare Equipment**

Gather a multimeter.

Ensure the board is powered off before connecting any probes.

## 2. **Check for Shorts**

Set the multimeter to continuity mode.

Check for shorts between the following pairs:

- 327+5V (C47-1, C48-1, C49-1, C72-1, C74-1) and GND (J4-10, J4-11, P1-10, etc.)

- 327+3V3 (C1-1, C2-1, C3-1, etc.) and GND

- 317+3V3 (P1-1, P2-2, P2-4) and GND

- 327+3V3_RF (C12-1, C25-1, C26-1, etc.) and GND

## 3. **Measure Voltage Levels**

Power on the board.

Measure the following voltage rails:

- **327+5V**: Expected 5.00 V ±0.10 V

- Probe at: C47-1, C48-1, C49-1, C72-1, C74-1, R17-2, R23-2, U12-4, U12-6, U6-14, U6-15, U6-16, U7-10, U7-2, U7-9, U8-5

- **327+3V3**: Expected 3.30 V ±0.10 V

- Probe at: C1-1, C2-1, C3-1, C4-1, C5-1, C50-2, C51-2, C52-2, C53-1, C54-1, C58-2, C59-2, C6-2, C68-1, C69-1, C7-2, C70-2, C71-1, C8-2, L10-2, R1-2, R19-2, R3-2, R33-1, R5-2, R6-2, R7-2, R9-2, U1-1, U1-13, U1-19, U1-32, U1-48, U1-64, U10-5, U10-8, U11-7, U11-8, U13-15, U13-2, U5-3, U5-8

- **317+3V3**: Expected 3.30 V ±0.10 V

- Probe at: P1-1, P2-2, P2-4

- **327+3V3_RF**: Expected 3.30 V ±0.10 V

- Probe at: C12-1, C25-1, C26-1, C27-1, C28-1, C34-1, C73-2, U12-1, U12-2, U2-14, U2-24, U2-3, U3-6, U4-6

## 4. **Record Results**

Document the voltage readings for each rail.

Ensure all readings are within specified tolerances.

## 5. Firmware Programming

## 1. **Prepare for Programming**

Ensure the board is powered on and connected to the programming interface.

Verify that the necessary firmware files are available.

## 2. **Connect Programming Interface**

Use the appropriate programming tool (e.g., JTAG or serial).

Connect to the following pins:

- JTAG: P1-5 (JTDI), P1-7 (JTMS_SWDIO), P1-9 (JTCK_SWCLK), P1-3 (JTRST)

- Serial: If applicable, connect to UART pins (TX/RX).

## 3. **Upload Firmware**

Follow the specific instructions for the programming tool to upload the firmware.

Monitor the upload process for any errors.

## 4. **Verify Firmware Installation**

After programming, perform a verification step to ensure the firmware is correctly installed.

Check for any error messages during the verification process.

## 6. Functional Test

## 1. **Initial Setup**

Ensure the board is powered on and all connections are secure.

Prepare any necessary test equipment (e.g., oscilloscope, signal generator).

## 2. **Perform Functional Tests**

Test basic functionalities such as:

- Power-on self-test (POST)

- Audio output functionality

- User interface responsiveness

Verify that all GPIO pins are functioning as expected by toggling them and checking the outputs.

## 3. **Check Communication Interfaces**

If applicable, test communication interfaces (I2C, SPI, UART):

- For I2C, check SCL and SDA lines for proper signaling.

- For SPI, verify MOSI, MISO, and SCK functionality.

- For UART, check TX and RX for proper data transmission.

## 4. **Document Results**

Record all test results, noting any issues encountered.

If any tests fail, troubleshoot the specific areas and retest as necessary.

## 5. **Final Review**

Review all documentation and ensure all tests have been completed satisfactorily.

Prepare the board for final inspection or deployment.