# Test Plan – Car Radio

Generated: 2025-09-27 14:24

# Test Plan PROCEDURE

## 1. Test Equipment

Power Supply, Generic, N/A, Adjustable voltage/current supply.

Digital Multimeter (DMM), Generic, N/A, For voltage and continuity measurements.

Oscilloscope, Generic, N/A, For frequency measurements.

JTAG Programmer, Generic, N/A, For firmware programming.

USB-TTL Cable, Generic, N/A, For serial communication.

Test PC, Generic, N/A, For programming and terminal access.

## 2. PROCEDURE

### 2.1. Visual Inspection

Inspect per IPC-610 Class 2.

### 2.2. Voltage Rail Checks

(Reference “Figure 4-1 Test Configuration A” when describing cabling.)

Set multimeter to diode/beep mode.

With black probe on ground pad (pin 2) of the input barrel jack (J3) (GND), verify the following locations ARE connected to ground:

P2 pin 1 (GPIO Header)   [generic]

With black probe still on ground pad (pin 2) of the input barrel jack (J3) (GND), verify the following are NOT connected to ground:

J3-1 (PWR_JACK)

U6-14 (+5V)

U6-15 (+5V)

U6-16 (+5V)

P2-2 (+3V3)

U12-1 (+3V3_RF)

With black probe on U6-14, U6-15, U6-16 (+5V), verify NOT connected:

P2-2 (+3V3)

U12-1 (+3V3_RF)

With black probe on P2-2 (+3V3), verify NOT connected:

U12-1 (+3V3_RF)

Instruct safe power-up using 5.0 V, 200 mA (default), warnings on over-current.

Connect supply to barrel jack via power cable. Reference Figure 4-1.

(WARNING: If UUT draws too much current, be prepared to turn off the power supply quickly.)

(WARNING: If current exceeds limit (constant-current mode), disable supply and stop procedure.)

With black probe on ground pad (pin 2) of the input barrel jack (J3) (GND), measure and record:

J3-1 (PWR_JACK): 5V ±100mV

U6-14 (+5V): 5V ±100mV

U6-15 (+5V): 5V ±100mV

U6-16 (+5V): 5V ±100mV

P2-2 (+3V3): 3.3V ±100mV

U12-1 (+3V3_RF): 3.3V ±100mV

Oscilloscope setup: CH1 1V/div, 5us/div, measurement=frequency.

Probe Y1 pin 1 and verify ~16 MHz.

Probe Y2 pin 1 and verify ~32 MHz.

Power off UUT.

Table – Voltage Rails to Check

| Item | Pin | Net | Expected | Probe Points | Measured / P/F |
| --- | --- | --- | --- | --- | --- |
| a. | C47-1 | 327+5V | 5.00 V (±0.10 V) | C47-1, C48-1, C49-1, C72-1, C74-1, R17-2, R23-2, U12-4, U12-6, U6-14, U6-15, U6-16, U7-10, U7-2, U7-9, U8-5 |  |
| b. | C1-1 | 327+3V3 | 3.30 V (±0.10 V) | C1-1, C2-1, C3-1, C4-1, C5-1, C50-2, C51-2, C52-2, C53-1, C54-1, C58-2, C59-2, C6-2, C68-1, C69-1, C7-2, C70-2, C71-1, C8-2, L10-2, R1-2, R19-2, R3-2, R33-1, R5-2, R6-2, R7-2, R9-2, U1-1, U1-13, U1-19, U1-32, U1-48, U1-64, U10-5, U10-8, U11-7, U11-8, U13-15, U13-2, U5-3, U5-8 |  |
| c. | P1-1 | 317+3V3 | 3.30 V (±0.10 V) | P1-1, P2-2, P2-4 |  |
| d. | C12-1 | 327+3V3_RF | 3.30 V (±0.10 V) | C12-1, C25-1, C26-1, C27-1, C28-1, C34-1, C73-2, U12-1, U12-2, U2-14, U2-24, U2-3, U3-6, U4-6 |  |

### 2.3. Firmware Programming

(Reference “Figure 4-2 Test Configuration B”.)

Connect JTAG Programmer to test PC via USB; connect to UUT JTAG.

Connect USB-TTL cable to test PC and UUT debug header (P2) with pinout:

- P2 pin 1 → GND (black)

- P2 pin 8 → Rx (white)

- P2 pin 10 → Tx (green)

Program the UUT per programming procedure. Verify success.

### 2.4. Functional Test

Open a serial terminal with parameters 115200 baud, 8 data bits, no parity, 1 stop bit (8N1).

Reset the UUT by pressing SW1.

Verify welcome screen prints.

Run commands: “bit.lora”, “bit.gps”, “bit.imu”, “bit.i2c”; verify each shows Pass.